| Architetture dei Sistemi di Elaborazione 02GOLOV | Delivery date: Tuesday 22/12 |
| --- | --- |
| Laboratory 9 | Expected delivery of lab_09.zip must include: - zipped project folder of the exercise 1 - this lab track completed and converted to pdf format. |

Solve the following problems by starting from the sample_BUTTON_LED project (open the file project from the uVision menu). Test the problems using the LandTiger emulator.

Remember to check if the emulator is enabled in the menu “Options for Target” after installing the emulator.

| Tab “Debug”: load the emulator’s library | Tab “C/C++”: define SIMULATOR |
| --- | --- |

Once you run the debug, if the emulator is correctly installed and added to the debug option, you shall find LandTiger under the Peripherals menu.

If you right-click on the board, you can access the configuration menu, where you can eventually change some default parameters (tab “Settings”)

Exercise 1) Implement an 8-bit “signed counter” by using LANDTIGER board; the software permits to use buttons to update a counting value which could be either positive or negative, and the LEDs to show the current value. By first using emulation capabilities, please implement the following functionalities:

- increment a variable every time the button  Key1 is pressed,

- decrement when Key2 is pressed (in case, go to negative number)

- reset the count when INT0 is pressed

LEDs are showing the current count in a binary, 2’s complement representation.

HINT: It could be useful to use a global variable in order to keep the information about turned ON LEDs. For example, using a variable called “char led_value”, already available in the project.

Q1: By adjusting the emulator settings, you can activate a non-ideal behavior of the buttons called "bouncing".  Do you notice any different behaviour on the emulator if you enable such a bouncing setting? Please comment.

Abilitando il bouncing, a causa dei rimbalzi nel valore di tensione misurato, il sistema rileva 2, 3 o 4 pressioni del pulsante invece che 1. Introducendo un sistema di temporizzazione che controlla il valore vero dei pulsanti ogni 50 ms si potrebbe risolvere questo problema.

Q2: What happens if you act on jumpers JP5 and JP8 with respect to the default configuration?

Se JP5 è disabilitato, il pulsante INT0 viene scollegato dal microcontrollore, pertanto nessun click di quel pulsante verrà registrato.

Se JP8 è disabilitato, i LED vengono scollegati dal microcontrollore. I tasti continuano però a funzionare e ad aggiornare la variabile led_value. Riabilitando JP8, infatti, si vede che i LED assumono il valore corretto, corrispondente alla sequenza di tasti premuta.

Using the emulator, check the schematic and fill the following table.

| Component | Pull-up resistor name | Pull-up resistor size |
| --- | --- | --- |
| LEDs | RN3 e RN2 | 470 ohm |
| INT0 | R22 | 10k ohm |
| KEY1 | R25 | 10k ohm |
| KEY2 | R23 | 10k ohm |